# VSC90 Command List.

This document describes the command list implemented on the VSC90 product.

It may be easier to navigate this document via headings (ctrl+F if viewing word version).

## Command processing:

‘toupper’ from ctpye.h is called when commands are processed with ProcessCommand() function, and therefore all below mentioned commands are case insensitive.

For some reason, some commands were given an alternative command string. Where applicable these have been documented.

Yellow indicates firmware needs to change for V90 system.

Commands not relevant to V90

Note – Unrecognised commands will return “what is [command entered] ?” Then the number of characters command contained… followed by a beep…

Bootloader:

Note – The first step indicated by a “<” is to erase the target boards ‘application firmware’, leaving only the bootloader firmware and some empty program memory to dump the new ‘application firmware’. So please note that calling the BL#~ command and entering a board into its bootloadable state will erase its ‘application firmware’ whether you proceed to program (bootload) on new firmware or not.
The second “>” character indicates the application firmware erasure has completed, the board should then proceed into its bootloadable state.

| Command | Description |
| --- | --- |
| BL#=n | Puts the control board or attached boards via the I2C bus into their bootloadable state. Depending on the parameter n passed, the following occurs: n = No parameter passed = Main control board goes into bootload state. n = “LAMPTRAY” = Lamp tray board goes into bootload state. n = “XYSTAGE” = XY Stage goes into bootload state (Not present in V90). |

General commands:

| Command | Alt command | Description |
| --- | --- | --- |
| 5 | Vc? | Returns the VSC90 controller firmware build number. |
| DATE | Vd? | Returns FW build date |
| Type |  | Returns product type, i.e. VSC90 |
| Baud=n |  | Sets I2C comms rate. From the PIC18F67K22s data sheet, it seems we are setting our I2C master clock out rate as: Fosc4*(n+1)=I2C clock Since SSPM register of SSPxCON1 register is set to 0x08 (0b1000) [page 284 of PIC18F87k22 datasheet] Firmware defaults n (SSPxADD) to 80. Thus unchanged, I2C clock: 32,000,0004*(80+1)=98,765Hz I2C clock For a 100kbit/s I2C clock: 32,000,0004*(79+1)=100,000Hz I2C clock Therefore Baud=79 should be used. |
| Test |  | Writes to I2C address 0xAA, if write fails it retries up to 7 times (8 tries total), then returns status. |
| ENUM *Need to add V90 addresses to be pinged* | EN? | Returns a comma separated list of connected PCB’s on the I2C bus. Error indicates a response from address 0x13. |
| Sstatus | S?? | Motor fault bits 0x01 Camera filter motor not yet home. 0x02 V 80 Close up lens motor not yet home. (Back Focus in V800) 0x04 One or more motors are moving. 0x08 V 80 Coax motor not yet home. (Not used in V800) 0x10 Camera filter motor fault. 0x20 V 80 Close up lens motor fault. (Back Focus in V800) 0x80 V 80 Coax motor fault. (Not used in V800) |
| Sn=n |  | FCB camera indicator beeps for completed ack + command and completed IRQ. 0=Disable, 1=Enable. |
| Se=n |  | Enable/disable error beeps for FCB camera error codes 0=Enable, 1=Disable |
| M?? |  | Check if motors are moving? 0 = Not moving 1 = Moving |
| XX |  | Reads PORTB RB2 pin val and returns added to 0x30 (0b0011,0000). Not sure why add the 0x30? There is another XX command one level down the if else chain in the command parser, it seems maybe these XX commands were just for firmware testing / debugging, so will remove unless someone tells me otherwise… |
| Lstatus | Y?? | Returns flap and safety relay states (open=0, closed=1). 0x01 Front flap sensor. 0x02 Left flap sensor. 0x04 Right flap sensor. 0x08 Debugging - Ignore bit 0x10 Debugging - Ignore bit 0x20 Debugging - Ignore bit 0x40 Safety relay #1. 0x80 Safety relay #2. Unsafe condition is 0, safe condition is 1 (hence 0b11XXX111 = all OK) |
| OFF Need to add new boards ‘off’s to this as they are added. | IF# | Turns all LEDS/Lights off. |
| FCB |  | Will return the last reply from the camera, which is retained by the V90controller. If this buffer has already been read (it usually has in most cases), zero will be returned. Reading this value at 1 second intervals at start up may indicate when the TWIGA board has completed sending VISCA initialisation commands. Wait for 2 consecutive zero responses. |
| TO=n |  | Set camera timeout (how long to wait for a reply from a VISCA command before ‘timing out’) TO=n, the units of n are 65 milliseconds. The default setting is 8 (8*65 ~= 0.5 seconds). |
| IN |  | Check if INTPC_ON3v3, to see whether internal PC is on/off. 0 = ON 1 = OFF |
| CE |  | Com error, if value is 0 no errors have occurred, otherwise indicates number of coms errors. |
| ICR Is this command still correct for current camera blocks used? -> 8101040102FF |  | Turns on the ICR (near infra red) mode. Returned packet should be 9041FF9051FF Ack followed by completion packet. |
| Beep |  | Beep takes three parameters, t, len, rest. // @t is us half period (of note). // @len is length in us (to play the note). // @rest is time after note is played to do nothing. |
| Beepx |  | Plays Bach in beep tones… |
| Gate This is setting pin GPA7 which is bypass red on the spot LED, not doing what its supposed to or board has changed… Gate enable is on GPA3… |  | Enables/disables gate enable on Spot LED’s IO expander |
| GE |  | Global enable, GE=1 enables, Ge=0 disables |
| Sync |  | Reads sync signal input and returns level (1=High, 0=Low) |
| Z#~ |  | Resets main V80 V90 microcontroller (PIC). Power will be removed from the hub after “KILL_TIME” number of ms or 250 ms, whichever is larger. |
| KT=n |  | Write new value of “KILL_TIME” stored in eeprom. New value = n. 2 bytes of EEPROM so max time of 65,535ms |
| KT? |  | Read value of “KILL_TIME” stored in eeprom. |
| KK |  | KK command kills machine (shuts it down) after KT number of milliseconds, OR after 250ms, whichever is longer. Has effect of holding down push button for 2-3 seconds. Kills PIC3v3 line, thus kills PIC. |
| K3 |  | Controls +3V3 2A output rail. K3=0 Enabled K3=1 Disabled |
| K5 |  | Controls +5V 5A output rail. K5=0 Disabled K5=1 Enabled |
| KC |  | Kc=1 Camera power on Kc=0 Camera power off |
| UT |  | Ut=1 FTDI interface on Ut=0 FTDI interface off |
| U2 |  | U2=1 USB 2 hub enabled U2=0 USB 2 hub disabled |
| U3 |  | U3=1 USB 3 hub enabled U3=0 USB 3 hub disabled |
| INT |  | INT=0 Internal PC Power Disabled INT=1 Internal PC Power enabled |
| INTEXT |  | Checks if external PC is connected. If not, tries to power and then check if internal PC is connected. Number of beeps indicates which PC is connected and controlling system. 1 = External PC 2 = Internal PC 3 = No USB detected (neither PCs detected) |
| EnableShutdown |  | Sets a variable ‘enable_shutdown’ that is checked in PICs high priority interrupt. Within the interrupt the PIC sets the INTPC_PB pin (RA6) High or Low based on the value of enable_shutdown (internal PC power push button pin). 1 = Shutdown Internal PC on high priority interrupt 0 = !Shutdown Internal PC high priority interrupt |
| PB |  | Directly sets the internal PC power pin (via the connected PIC pin) doing the following to pin RA6: pin High, set pin as output, set pin High again… , wait 250ms, set pin Low. |
| PC |  | Reads external PC Vbus level, Internal PC Vbus level and then internal Vcc level, all through 3.3v level shifters. N channel inverters so logic is flipped, 0=ON, 1=OFF. Returns ON/OFF values in order: - ExtPCVbus - IntPCVbus - IntPCVcc |
| ReadVolts |  | Returns an integer value in millivolts (mV) indicating the bus voltage. |
| ReadAmps |  | Returns an integer value in milliamps (mA) indicating the current draw |
| ReadWatts |  | Returns an integer value in milliwatts (mW) indicating the power |

Flood Arrays (L & R) 260-022:

| Command | Alt command | Description |
| --- | --- | --- |
| PWMP=n |  | Reinitialise the PCA9685PW pwm, GPIO expander on the flood array boards (LEFT and RIGHT) setting PWM pre-scaler = n. Pre-scaler n: 25,000,0004096*n-1=PWM frequency Default pre-scaler value = 7: 25,000,0004096*7-1=870Hz Minimum pre-scaler = 3 (1,526Hz) Maximum pre-scaler = 255 (24Hz) |
| PWMF=n |  | Reinitialise the PCA9685PW pwm, GPIO expander on the flood array boards (LEFT and RIGHT) setting PWM frequency = n. Frequency n: 25,000,0004096*n-1=Prescaler Default pre-scaler value = 870Hz: 25,000,0004096*870-1=7 Minimum frequency = 1,526Hz (pre-scaler of 3) Maximum frequency = 24Hz (pre-scaler of 255) |
| UV2=n *SEE BELOW NOTE |  | Turns on the three additional UV LEDs at intensity n. (λ = 385nm) Flaps and safety relay contacts must be ‘made’ to turn UV LEDs ON. |
| IR780=n IR850=n IR940=n IR980=n W1=n W2=n OVD1=n Will rename/move OVD2=n Will renam/move In V80 OVD commands control OVD LEDs, I will rename/move these commands in future to be common. |  | Turns on two LED’s (per side) at intensity n. Intensity is passed as 0-1023 value, a value below 100 will turn the light source off. NOTE – W1, W2 (white floods), OVD1, OVD2 and UVLED3 (one of 3 UV LEDs switched on when UV2=n command is used) are all driven by a common driver. This driver can drive up to 2 LEDs at any one time. Firmware prevents more than 2 being switched in by turning off all 5 LEDs if a command is passed that would switch more than 2 LEDs on. |

Flood Arrays MS (V900, not used on V90)

| Command | Alt command | Description |
| --- | --- | --- |
| RESETFLOOD |  | Reinitialises the PCA9685PW pwm, GPIO expander on the flood array boards (LEFT and RIGHT). |
| IRW *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 |  | IRW 110011 n Turns on a combination of IR and white floods LEDs using a binary sequence of 6bits. The intensity value n is common for all LEDs and will accept values 0-1023. The order of bits from left to right is as follows: • 780nm • 850nm • 940nm • 980nm • White 1 • White 2 For example, the command to turn on 940nm, 980nm, White 1 and White 2 at intensity 400 would be: IRW 001111 400 |
| IR3D *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 |  | Used to switch on single IR LEDs in the four corners of the VSC900, primarily for generating IR source images for the 3D surface mapping tool. Turns on a single IR LED n at intensity x. The intensity is on a scale of 0-1023. The wavelength index is shown in the table below. LED No Wavelength (nm) Corner Location 1 980 Left Front 2 940 Left Front 3 850 Left Front 4 780 Left Front 5 980 Left Rear 6 940 Left Rear 7 850 Left Rear 8 780 Left Rear 9 980 Right Rear 10 940 Right Rear 11 850 Right Rear 12 780 Right Rear 13 980 Right Front 14 940 Right Front 15 850 Right Front 16 780 Right Front *Corner location assumes that the user is looking into the machine from the front flap. |
| UV365 *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 | UV3 | Turns on the UV LEDs n at intensity x. The intensity is on a scale of 0-1023. The LED positions are shown in the table below. Index (n) Led No Mode 1 1 Single 2 2 Single 3 3 Single 4 1, 2, 3 Flood Mode 5 1, 3 Left and Right Mode Typically, n=4 for flood lighting, the other options are primarily intended for test/diagnostic purposes. |
| FloodDimWhite *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 | FDW | Dims only the white LEDs on flood PCB. Intended to be used to trim light level once a light source has already been switched on. Particularly when used as fill lighting with UV light sources. |
| FloodDimAll *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 | FDA | Dims all the flood PCB light sources (White, IR, UV and MS). Intended to be used to trim light level once a light source has already been switched on. |
| MS *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 |  | Turns on a multi spectral LED n at intensity x. The intensity is on a scale of 0-1023. The wavelength index is shown in the table below. LED No Wavelength (nm) Filter Driver 1 365 VIS/UV UV 2 385 UV MS 3 405 VIS MS 4 425 VIS MS 5 450 VIS MS 6 475 VIS MS 7 500 VIS MS 8 530 495 MS 9 560 495 MS 10 600 550 MS 11 620 570 MS 12 640 590 MS 13 660 610 MS 14 680 630 MS 15 700 665 MS 16 730 695 MS 17 750 695 MS 18 780 725 IR 19 810 780 MS 20 850 780 IR 21 890 850 MS 22 910 850 MS 23 940 850 IR 24 980 850 IR 25 1050 850 MS 365nm (Index 1) is identical to the original UV365 command, however only 2 of the 3 UV LEDs are turned on. |
| MSOFF *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 |  | Turns off all MSFlood boards light sources |
| MSWhite *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 |  | Turns on a white LED n at intensity x. The intensity is on a scale of 0-1023. This command is primarily intended for testing/diagnostic purposes. The LED positions are shown in the table below. Index (n) Led No Mode 1 1 Single 2 2 Single 3 3 Single 4 4 Single 5 5 Single 6 6 Single 7 7 Single 8 1, 7 Flood Mode 9 1, 2, 3, 4, 5, 6, 7 All On Mode |
| MSSIDE *A this currently controls the multispectral board used on v900, I believe we will need a similar function that controls the flood PCBs used on the v90 |  | Selects left, right or both sides of the multispectral flood lighting for testing purposes. The table below shows the valid n values. Index (n) Side/s 0 Both (default) 1 Left 2 Right By default, both multispectral flood PCBs will be used when any flood or multispectral flood commands are used. However, for testing purposes it may be desirable to drive one flood array at a time. To do this an MSSIDE 1 or 2 command should be sent to select the side, followed by the lighting commands for that side. MSSIDE 0 should then be used to return to the default state. Additionally using the OFF (If#) command will also return the flood arrays to the default state of both being enabled. |
| PWM |  | Directly sets the PWM from 0-100% with an accuracy ~ +/-1% of requested duty cycle. |
| PP=n1 n2 If this is even needed, then should be changed so it checks the pin trying to be controlled, and if its not an LED pin, don’t PWM that pin… |  | Control PWM driving LED n1 with duty cycle n2. LED’s (pins of PCA9685 IO expander) LED0->15 Not all pins on IO expander are LED’s so be careful not to PWM a non LED pin… LED0 = IR850 LED1 = White LED2 = IR780 LED3 = UV LED4 = IR940 LED5 = IR980 LED6 = Not Connected LED7 = Gate_Enable (NOT AN LED) LED8 = Flood white1 LED9 = Flood white 2 LED10 = OVD LED 1 LED11 = OVD LED 2 LED12 = UV LED 3 LED13 = Not Connected LED14 = Not Connected LED15 = DAC_EN (NOT AN LED) |
| LQ? n |  | Diagnostic command to read the EEPROM intensity level stored for a specified OVD LED index. For example, Lq? 12 might return the value 500 if that was the last specified intensity. Range of 1 -> 52 (min -> max OVD LED’s) |
| LQ=n1 n2 |  | Passed 1 parameter, number of OVD LED to turn on. Turns that LED on. Passed 2 parameters, turns on n1 number LED and sets intensity to n2 value (0-1023 for 0-100%) |
| Soak |  | Runs a soak test turning on all control board controlled light sources, motors etc for testing. |
| TransSoak |  | Runs a soak test turning on all light sources etc controlled by the translight board |

Front OVD Array commands:

| Command | Alt command | Description |
| --- | --- | --- |
| FOVDSHIFT=n |  | Changes all the I2C addresses for the Front OVD PCB to their values before any XOR shifting is applied on the lamp tray PCB. This allows the FOVD PCB to be directly plugged into the control PCB for testing purposes. The table below shows the valid n values. Index (n) Shift Removed 0 No (default) 1 Yes By default, the FOVD PCB addresses will always be shifted to be compatible with the Lamp Tray PCB. This default state will be restored after a reset to power-cycle. |
| ResetFOVD | RFOVD | Reset the front OVD boards IO expander (MCP23017) to initial values (all OFF). Returns 0x40 on successful reset. |
| ReadFOVD | RDFOVD | Returns two strings of 0’s or 1’s representing the OLAT (Output) registers of PORT A and PORT B of the front OVD’s IO expander (MCP23017). 1 = High 0 = Low Ordered so that PORTA is printed, followed by PORTB. Bits represent pin 0->7 from left to right. |
| SetFOVD=n1 n2 | SFOVD | Set front OVD LED number n1 to brightness level n2. Brightness level is passed as a 0-1023 value. There are 11 LEDs, thus n1 should be a value 1-11. The front OVD board has 9 brightness levels from OFF to full brightness. For this reason, the 0-1023 value passed results in the following: n2; Brightness. 0-99; OFF (Clears LED). 100-199; Level 1. 200-299; Level 2. 300-399; Level 3. 400-499; Level 4. 500-599; Level 5. 600-699; Level 6. 700-799; Level 7. n2>800; Level 8 (Max). Note – Designed to have 1 LED illuminated at any time. If an LED is selected to be turned on with a brightness > 99, the LED driver will be enabled. Otherwise, it is disabled. |

Spot Light Array 260-023:

| Command | Alt command | Description |
| --- | --- | --- |
| RESETSPOT |  | Reinitialise the spot LED array board, effectively resetting the boards: - MCP23017 GPIO expander - LTC2633 DAC - TMP112 temperature sensor |
| Spot n1 n2 |  | Turns on violet and blue spots LEDS together at intensity n2. There must be exactly ten 1s and 0s in the first argument string (n1) to turn on any number of LEDs corresponding to the LED order from left to right of: Violet, Blue, Blue-Green, Cyan, Green, Amber, Red, Deep Red, Far Red and White. If n=0, LEDs will turn off. If more than 2 LEDs are on, the intensity level will be automatically de-rated, depending on the number of active LEDs. The maximum de-rating factor for all 10 LEDs is 0.5. The actual intensity applied will be returned. If P4 is subsequently used to adjust the intensity, the applied intensity, de-rated, if necessary, will also be returned. The de-rating factors are, for 1 to 10 LEDS: 1.0, 1.0, 0.90, 0.80, 0.75, 0.70, 0.65, 0.60, 0.55, 0.50 LED wavelengths are: Violet 410nm Blue 445nm Blue/Green 475nm Cyan 500nm Green 520 nm Amber 590nm Red 640nm Deep Red 660nm Far Red 740nm White 420nm-700nm |
| VIOLET n |  | Turns on violet spot LED only at intensity n. (0-1023) |
| BLUE |  | Turns on blue spot LED only at intensity n. (0-1023) |
| BLUEGREEN |  | Turns on blue/green spot LED only at intensity n. (0-1023) |
| CYAN |  | Turns on cyan spot LED only at intensity n. (0-1023) |
| Green |  | Turns on green spot LED only at intensity n. (0-1023) |
| Amber |  | Turns on amber spot LED only at intensity n. (0-1023) |
| Red |  | Turns on red spot LED only at intensity n. (0-1023) |
| DeepRed |  | Turns on deep-red spot LED only at intensity n. (0-1023) |
| FarRed |  | Turns on far-red spot LED only at intensity n. (0-1023) |
| WhiteSpot |  | Turns on white spot LED only at intensity n. (0-1023) |
| SPOTOFF |  |  |
| SPOTSHIFT n |  | Changes all the I2C addresses for the Spot PCB to their values before any XOR shifting is applied on the lamp tray PCB. This allows the Spot PCB to be directly plugged into the control PCB for testing purposes. The table below shows the valid n values. Index (n) Shift Removed 0 No (default) 1 Yes By default, the spot PCB addresses will always be shifted to be compatible with the Lamp Tray PCB. This default state will be restored after a reset to power-cycle. |
| P4=n |  | Sets spot board intensity to value n, 0-1023. (Sets DAC on spot PCB) |
| SHP |  | Returns a 10-character string. '1' indicates that the spot wavelength LED is fitted and '0' that it’s not. Values read from main control board PIC’s EEPROM, where list of fitted spotlights are stored, 1 byte per light source (10 bytes) from bytes 190-200. |
| SETSPOT=n* |  | Setup available spot LEDs, default is 1. Accepts a string n* of characters that are 1’s or 0’s to set spotlights as: 0 = unavailable 1 = available Writes values to main control board PIC’s EEPROM memory bytes 190 – 200 (1 byte per spotlight). |
| Temp |  | Get temperature from TMP112 sensor on spot PCB |

Lamp-tray 284-021:

| Command | Alt command | Description |
| --- | --- | --- |
| Lver Should probably update this and the lamp tray firmware to read and report V90_LAMPTRAY if different. |  | Get lamp tray version, will return a different string for the detected mode of a connected lamp tray board: Boot load mode = “BOOTLOAD” Application layer = “VSC900_LAMPTRAY” Undetermined mode = “?” |
| Lbuild | VL? | Get 3-byte build number from lamp tray |
| Ldate |  | Get 4-byte date stamp from lamp tray |
| Lexe |  | Write all 0’s to lamp-tray (all off) |
| OVDCONF |  | Pings I2C addresses for the various configurations of lamp tray and flood array board combinations and returns the 8-bit unsigned configuration byte which indicates those boards which were found. Bit 0 = Left MSFLOOD_GPIO V900 Left flood Bit 1 = Right MSFLOOD_GPIO V900 Right flood Bit 2 = Left_PCA9685 LED driver (GPIO with PWM) V80 Left flood Bit 3 = Right_PCA9685 LED driver (GPIO with PWM) V80 Right flood Bit 4 = V900 Lamp tray board Bit 5 = V80 Lamp tray board Bit 6 = V90 Lamp tray board Bit 7 = Currently unused. |
| COAX n | Ax=n | Turns on the Coax LED at intensity n |
| SIDELEFT n | Ls=n | Turns on the left side White and IR LEDs together at intensity n. |
| SIDERIGHT n | Rs=n | Turns on the right side White and IR LEDs together at intensity n. |
| Side n1 n2 |  | Turns on one or more side lights at intensity n2. Parameter n1 is a string of exactly four 1s or 0s which controls the side lights in the order: Left White - Right White - Left IR - Right IR |
| UV255 n | UV1=n | Short wavelength UV LEDs. The wavelength will be 255nm for the VSC90/900 UV LEDs. UV LEDs are either on (n != 0) or off (n = 0) |
| UV308 n | UV2=n | Medium wavelength UV LEDs. The wavelength will be 308nm for the VSC90/900 UV LEDs. UV LEDs are either on (n != 0) or off (n = 0) |
| P5=n |  | Sets lamp tray board intensity to value n, 0-1023. (L & R side lights, Coax, OVD on Lamp tray) (Sets PWM duty cycle *PWM Output from PIC* which drives brightness level pin of LT3791 LED driver buck-boost). |

Trans Light 284-027:

| Command | Alt command | Description |
| --- | --- | --- |
| Transinit | TI | Re-initialises the translight ready for use. Returns 0 if the translight tray can be found for initialisation. This command should be followed with a TRANSCHECK to make sure the translight is ready for use. |
| TransCheck *This will need changing to v90 translight I2C addresses from V900 | TC | Pings I2C address of the translight, if the translight GPIO responds with an acknowledge, it is connected, otherwise returns ‘2’ to indicate the tray is missing (not connected). Also then checks the output status of the GPIO pins. If they are correct (translight GPIO initialised correctly) ‘0’ is returned. If not, it suggests the translight has been swapped and I/O are not correctly set in their default states, so ‘1’ is returned. |
| TransSpot=n1 n2 | Ts=n1 n2 | Turns on a combination of IR and white transmitted spot LEDs using a binary sequence of 4bits (n1) at the selected intensity level n2. The intensity value n2 is common for all LEDs and will accept values 0-1023. The order of bits from left to right is as follows: • 780nm • 940nm • 850nm • White For example, the command to turn on 780nm, 940nm and White at intensity 500 would be: TRANSSPOT 1101 500 |
| TransFlood=n1 n2 (TF=n1 n2) | TF | Turns on a combination of IR, UV and white transmitted flood LEDs using a binary sequence of 4bits (n1) at the selected intensity level n2. The intensity value n2 is common for all LEDs and will accept values 0-1023. The order of bits from left to right is as follows: • UV 365nm • 780nm • White • 850nm Note: Not all combinations valid. If the UV365nm bit is set all remaining bits will be ignored. For example, the command to turn on 850nm, White at intensity 500 would be: TRANSFLOOD 0011 500 |
| TransOff | T0 | Sets translight to initial state (all off) |
| PDAC=n |  | Enables or disables flood array (Left and right) DAC’s. 0 = Disable 1 = Enable |

Anti Stokes LED Driver 260-038:

| Command | Alt command | Description |
| --- | --- | --- |
| ASVER |  | Returns the version of antistokes fitted in the machine. - 0 – Not Fitted - 1 – Flash Tube Antistokes (obsolete) - 2 – LED Antistokes |
| AS0 |  | Switch off the LED antistokes array. Set outputs to initial state and enable DIM output to DAC (dims output of DAC). See ASDIM for details on how dimming works. |
| AS1 |  | Switch on bank 1 of the antistokes LED array in low power mode. AS0 command should be used first to turn off the array if it was previously used in high power mode. |
| AS2 |  | Switch on bank 2 of the antistokes LED array in low power mode. AS0 command should be used first to turn off the array if it was previously used in high power mode. |
| ASDIM | ASD | Sets antistokes array in ‘Dim’ mode, reducing intensity. AS1 and AS2 must have been sent prior to this command. Dimming of DAC is a bit convoluted: The GPIO output is set low, this means a higher voltage is output from the OPAMP (OVERTEMP) used to sense/buffer the temperature sensing thermistor on the AS array. That in turn switches on an N-FET (more) creating a divider, thus lowering the voltage on the CTRL pin of the LT3756 LED driver… meaning less output current… |
| ASHIGH | ASH | Switch on the antistokes array in high power mode. AS1 and AS2 must have been sent prior to this command. |
| ASREAD |  | Reads and returns the port register of the IO expander on the AS driver board (MCP23009). Returns 8bit unsigned integer where each bit represents IO pins state (High/Low). For example, 10010001 would mean pins GP7, GP4 and GP0 are high, other pins are low. |
| AS? |  | Returns 1 if the Anti-Stokes unit is fitted, 0 if not. Reads main control board PIC’s EEPROM memory byte 200. 0 = not fitted, 1 = fitted |
| SETSTOKES=n |  | Writes byte n to Anti-Stokes fitted/not fitted memory byte. 0 = not fitted, 1 = fitted Memory byte 200 of main control board PIC’s EEPROM |

Anti Stokes Flash:

| Command | Alt command | Description |
| --- | --- | --- |
| Charge |  |  |
| FLPP | FL+ |  |
| FLMM | FL- |  |
| FT |  |  |
| FSTATUS | FL? |  |
| FD=n |  | Set flash delay number n |
| FD? |  | Read and return current flash delay number n |
| Flash | FL@ | Fires the flash, then freezes the frame after the number of frame counts set by Fd=n . Use Fl- to turn freeze frame off. Fd is set to default to 1, which probably works. If not try different values. The flash function should return zero. If non-zero is returned it indicates that the VSC80 could not detect the camera's horizontal sync pulse and a timeout condition was detected. |

Phosphorescence:

| Command | Alt command | Description |
| --- | --- | --- |
| LD=n |  | Sets the LED off frame delay number. Number of frames to wait (count) before freezing the video. |
| LD? |  | Reads and returns the LED off frame delay number. |
| GD=n |  | Sets LED on delay number. After camera frame sync pulse, number of milliseconds before all LED’s are switched off via global enable/disable signal. |
| GD? |  | Reads and returns the LED on delay number |
| LL@ Command calls functions controlling MSflood board (v900 floods), these will need to be changed to function with V90 flood boards (V80 flood boards…) |  | LL@ (Phosphorescence?) Waits for the next camera frame sync pulse then delays by the number of milliseconds specified by Gd=n, then switches off all LEDS with a Global Enable signal. This function will capture and freeze the video after a further frame count set by Ld=n. Where n can be 0, 1, 2... selected by experiment. This function should return zero. If non-zero is returned it indicates that the VSC80 could not detect the camera's horizontal sync pulse and a timeout condition was detected. After use, you may need to re-enabled Global enable with Ge=1 to turn LEDs back on, unless there is a LED change or LED on command, which should automatically re-enable the Global enable. You may also need to clear the controller buffer which holds the VISCA reply packet using the command FCB, which returns and clears the return buffer in the microcontroller. For normal VISCA commands, the buffer content is returned and cleared automatically. Gd? and Ld? Will return the above settings. One frame is 40 milliseconds. Gn should not normally exceed 40. The camera frame rate may need to be 1/25 (40 milliseconds) for Anti-Stokes and phosphorescence to operate correctly. |

Camera Filter Wheel [motor] (controlled by 278-022):

| Command | Alt command | Description |
| --- | --- | --- |
| SS=n |  | Sets start speed as n value*10. This sets a variable ‘motorspeed’, which in turn sets timer registers for Timer 0 on the PIC. This appears to be set up as a 16-bit timer, so the n*10 is likely a case of converting a 0-100 (%) input value for n, into a ~0-1023 10 bit setting of the timer registers (how many counts until an interrupt occurs). The basic operation seems to be: - Wait until Timer 0 overflows and an interrupt is generated. - In interrupt service routine: - Disable timer. - Reset timer high and low register to 8LSB’s and 8 HSB’s of ‘motor speed’ - Re-enable Timer 0. - Move motor 1 step in direction (set externally) - speed (timer count, previously set by ‘motorspeed’) is increased or decreased depending on whether acceleration or deceleration is required. In turn this reduces timer register values, thus time between next step is increased/decreased. - Return from ISR. |
| ES=n |  | Sets ENDSPEED, this is the ‘high limit’ for the value of speed, speed being the time between steps and therefore higher value of speed = slower rate of steps (motor turns slower). |
| ST=n |  | Sets step speed, number to increment or decrement ‘speed’ (time between each step taken) Startup default = 45. |
| DE=n |  | Sets DEAC (Deceleration), a value which is compared against the number of steps to move ‘move’. If the value of ‘move’ is less than the Deceleration value, speed (time between steps) is decremented by STEPSPEED. This effectively decreases time between steps, increasing motor speed. Startup default = 100. ‘DEAC’ set by DE=n can simply be thought of as a limit value, which when hit, starts deceleration. For example, move 100 steps, DEAC = 50, speed will increase until max (or step 50 whichever comes first), then deceleration will start. |
| CNH |  | Home all motors; Filter wheel, Zoom Lens and Coax Motor. |
| C1=n |  | Where n = (0-12), moves the filter wheel to position n, by the shortest route. Returns 0 if successful, 1 if the filter position was out of range. |
| C1? |  | Returns current filter wheel position. |
| C1z |  | To calibrate: Apply C1z to clear the calibration. |
| C1h |  | Apply C1h to home the wheel. |
| C1> |  | Use C1> or C1< as required, to nudge the wheel either direction to centralise the clear filter element in the field of view. This adjustment value is stored in EEPROM Finally apply C1h to home the wheel, and verify that the alignment is OK. C1h can be applied thereafter anytime. Motors must be homed from a home command at power on. |
| C1< |  |  |
| C1O |  | C1o is a diagnostic to read the offset value and will return a signed number. |
| CQ |  | Set filter motor to move n number of steps at the current speed (set by ST=n). Positive value = Forward direction Negative value = Reverse direction |
| C1X |  | Reads and returns camera filter wheel home position hall effect sensor. This needs checking, assumption made on logic 0 = Home 1 = !Home |
| C1S=n |  | Enable/Disable Camera Filter stepper motor driver sleep mode (20uA max instead of 4-8mA) 1 = Awake 0 = Sleep Mode |
| C1L |  | Control high or low power mode of operation for camera filter wheel stepper motor driver (via LP_C of PIC to DRV8811PWP): 0 = Low power mode disabled, 0.785A drive current 1 = Low power mode enabled, 0.305A drive current |
| C1F | C1P | Returns filter population string. 1 = filter element is fitted, 0 = filter element blank. (future use). Note first V80s (before M2 added), returned fixed string "1111111111111" Reads main control board PIC’s EEPROM, bytes 164 – 190 (26 bytes total), |
| SETFILTER=n* |  | Setup active camera filers, defaults = 1. Command accepts a string of up to max number of filters (MAXFILTERPOS) number of characters, that are 1’s or 0’s to indicate fitted or not fitted filters. It writes these fitted/not fitted values to bytes 164 – 190 of the main control board PIC’s EEPROM. |

Closeup lens [motor] (controlled by main control board 278-022):

| Command | Alt command | Description |
| --- | --- | --- |
| RESETLENS |  | Reinitialise the PCA9685 that used to control the lens motor. If no longer used, this command needs removing, otherwise needs changing to how motor is currently being controlled (with DRV8811PWP, which is in turn controlled directly by PIC pins). |
| C3? |  | Returns close-up lens position. |
| C3=n |  | Move lens in or out of optical path of camera. 0 = Move out of optical path (to home) 1 = Move into optical path |
| C3h |  | Move out of optical path (to home) |
| C3> |  | Use C3> or C3< as required, to nudge the lens motor in either direction to calibrate the lens home position. This adjustment value is stored in EEPROM. Apply C3h to move close-up lens to the calibrated ‘home’ position and verify that the alignment is OK. C3h can be applied thereafter anytime. Motors must be homed from a home command at power on. |
| C3< |  |  |
| C3z |  | Applies 80 degree offset from the above calibrated ‘home’ position (335 steps) and saves that to EEPROM as a variable named LENS_OFFSET. 1.8° step angle motor, micro-stepping 8 micro steps per full step, thus 1.8/8 = 0.225° ‘micro-steps’. 335(usteps) * 0.225° = 75.375° |
| C3O |  | Reads and returns the current value of LENS_OFFSET |
| C3X |  | Reads and returns the status of the close-up lens hall effect sensor which indicates whether close-up lens is in its ‘home’ position. ‘Home’ is the lens not in the optical path of the camera. |
| C3S=n |  | Sets close-up lens stepper motor driver into sleep or not. Input to the driver is !SLEEP, so for n= : 0 = Motor driver in sleep mode 1 = Motor driver awake |
| C3L=n |  | Control high or low power mode of operation for camera filter wheel stepper motor driver (via LP_B of PIC to DRV8811PWP): 0 = Low power mode disabled, 0.559A drive current 1 = Low power mode enabled, 0.153A drive current |

Coax mirror [motor] (controlled by Lamp Tray slave PCB 284-021 which receives commands from main control board 278-022):

| Command | Alt command | Description |
| --- | --- | --- |
| Calibration Note: | I2C between the boards. | Calibrate as for camera filter. Apply: C4z, C4h, then C4> or C4< as required to set horizontal. Apply C4=1, then C4p or C4m as required for a coax image (a suitable coax sample is required). Finally apply C4h (or C4=0) and C4=1 to verify both positions are correct. The coax lens will not move until it is calibrated. |
| C4? |  | Reads and returns the coaxin variable which is storing the position of the Coax mirror (in or out of optical path of camera). 0 = ‘Home’, Coax mirror out of cameras optical path 1 = Coax moved 45° from home, into the cameras optical path. |
| C4h |  | Home (parks) motor(horizontal), out of cameras optical path. |
| C4z |  | Clears both the calibration points. |
| C4=n |  | Moves coax mirror dependant on parameter n passed: 0 = Move to ‘home’ position (out of cameras optical path) 1 = Move to working position (45 degrees or as required) |
| C4PP | C4+ | Move coax mirror to working position (45 degrees or as required) |
| C4MM | C4- | Move coax mirror to home’ position (out of cameras optical path) |
| C4> |  | Fine adjustment to ‘home’ position (positive increment) |
| C4< |  | Fine adjustment to ‘home’ position (negative increment) |
| C4P |  | Fine adjustment of working position (presumably P for plus increment) |
| C4M |  | Fine adjustment of working position (presumably M for minus increment) |

Diagnostics:

| Command | Alt command | Description |
| --- | --- | --- |
| READ |  | Reads and returns byte stored at memory address ‘address’ which is set by command ADDR |
| WRITE=n |  | Writes byte n to memory address ‘address’ which is set by command ADDR |
| ADDR=n |  | Sets the ‘address’ variable which is used by above READ and WRITE commands when reading and writing to memory (I2C EERPOM). n = 1 byte address For the 24AA256SN 256kB EEPROM, memory is organised into 64-byte pages. When a byte is written, the whole page is written (refreshed), this should be considered with regards to “life” of the EEPROM. The current ReadMem and WriteMem functions only use the lower byte (8-bits) of the 15-bit address. In other words, the high byte is not used (all 0’s). This restricts the EEPROM usage from the full 256kB (2^15 = >32,000. 32,000*8 = 256,000 bytes) to (2^8 = 256. 256*8 = 2,048 bytes) |
| OUT=n1, n2, n3… |  | General output of I2C data for testing. Multiple bytes are taken as input parameters (8 max) and are used in the following way: Byte 1: I2C address to write to. Will be first byte transmitted once to the I2C bus. Byte 2, through 8: If the byte is an ‘r’ or ‘R’ character, then a byte will be read from the I2C bus and returned (if multiple bytes are read only the last byte will be returned). If the byte is any other value, it will be written out onto the I2C bus. For example, if you wanted to read the second byte (0x01) of the EEPROM with read address 0xA0 you could call this command with the following parameters: "160 0 1 161 r" Remembering that the read address 0xA0 = 160. The write address is 0xA1 = 161. High and low bytes to read address for second byte is 0x01. |
| I2C |  | Writes I2C address byte (n1) followed by data byte (n2). If the write of these bytes was successful returns 0, else returns a number indicating error encountered. Returned Value: -2 = NAK received (Not acknowledged) from I2C bus -1 = WCOL occurred (Write collision) |
| SPOTLATA |  | Read and return the Spotlight arrays OLAT registers, these are the registers of the IO expander that latch outputs logic HIGH or LOW. Warning, the returned values are what the OLATA register is set to, not what logic level the PORT is. The OLAT register could be HIGH (1) but the PORT (pin) could be LOW. 1 = request output HIGH 0 = request output LOW What is connected to PORTB of this GPIO expander (MCP23017): GPA7 Red 640nm GPA6 Deep Red 660nm GPA5 Far Red 740nm GPA4 White 420nm-700nm GPA3 GATE_ENABLE GPA2 !SPOT_EN |
| SPOTLATB |  | Read and return the Spotlight arrays OLATB registers, these are the registers of the IO expander that latch outputs logic HIGH or LOW. Warning, the returned values are what the OLAT register is set to, not what logic level the PORT is. The OLAT register could be HIGH (1) but the PORT (pin) could be LOW. 1 = request output HIGH 0 = request output LOW What is connected to PORTB of this GPIO expander (MCP23017): GPB5 Violet 410nm GPB4 Blue 445nm GPB3 Blue/Green 475nm GPB2 Cyan 500nm GPB1 Green 520 nm GPB0 Amber 590nm |
| SPOTLEDAX=n* |  | Set Spotlight array GPIO expander (MCP23017) OLATA registers. Input parameter n must be a string of 8 ones or zeroes representing High (1) or Low (0) for each pin to be controlled. |
| SPOTLEDBX=n* |  | Set Spotlight array GPIO expander (MCP23017) OLATB registers. Input parameter n must be a string of 8 ones or zeroes representing High (1) or Low (0) for each pin to be controlled. |
| XB=n |  | Set eeprom_block (page) to be read/written by other functions. eeprom_block is the 64byte page to be read (0-499) For some reason this can be set to a max of 510… Not sure why, as the memory is 256kbit, 32kbyte with 64-byte pages so 500 pages… 256kBit = 32kbyte = (64*500) = 32000 |
| XR? |  | Reads page (64 bytes) of data from external EEPROM starting at address eeprom_block *64. For example, read page 6, XB=6 command would be sent first, then XR. |
| EEPROMREADTEST |  | Functions similarly to XR? command described, but also writes the EEPROM data to the USB com port. |
| EEPROMWRITETEST |  | Test eeprom write command which writes one page (64 bytes), starting at address 0 (first byte of first page) of the EEPROM memory. Should write 0-63 to bytes 0-63 if write is successful. |
| XWPP | XW+ | Read 64 bytes from USB (com port), then write one page (64 bytes) to the external eeprom over I2C. The written bytes are then read and verified, and number of ‘bad’ (incorrectly written, non-verified) bytes is returned (written to USB com port). |
| XW=n* |  | Writes input string n* of up to 64bytes in length (64 ascii characters) to EEPROM page (64bytes) starting at byte address eeprom_block (set by XB=n command)*64. Verifies written bytes and returns (prints to USB com port) the number of incorrectly written bytes. |

Machine Configuration:

| Command | Alt command | Description |
| --- | --- | --- |
| GETMACHINE | OP? | Read EEPROM byte 156 of the main control boards PIC, designated to Machine type/build variant. Depending on value read, following is returned (via USB COM port): 0 = Machine Full 1 = M1 2 = M2 |
| SETMACHINE=n |  | SETMACHINE=n, where n is the value representing ‘build type’ to set machine to. For full specification machines n should be set to 255. For M1 machines, set n to 1, for M2 machines set n to 2. Machine type is written to control board PIC’s EEPROM byte 156. |

XY stage commands:

| Command | Alt command | Description |
| --- | --- | --- |
| N/A |  | Moveable XY stage not implemented on VSC90 |