VSC900 Commands

FTDI - USB interface to PC. Open the FTDI driver by Serial Number “VSC90” or “VSC900”.

This document outlines commands for the VSC900. However, many of these will also apply to the VSC90. Any VSC90 specific commands will be highlighted.

Where possible, a 3 character V8 mnemonic is given in BLUE. Either code can be used. In general, new VSC900 commands have an intensity value following the instruction. If this is omitted, the previous intensity is used. V8 commands can use, for example Rs+ (fully on), Rs- (off), Rs=n (intensity=n). The intensity range n is 0 (off) to 1023 (fully on).

Intensity can also be set with a separate instruction, however there are eight(!) intensity channels (2 on the transmitted  PCB, 1 on the lamp tray PCB, 1 on the spot PCB and 2 each on left and right flood PCBs)

In general, the V6/V8 did not have (this amount of) dimming on all channels, so these may be additional instructions. Intensities will be set to 50% (512) at power on.

The Commands listed below (case insensitive) should be terminated with a line feed (0x0a) character. Commands and argument(s) may be separated by a space, comma or  '=' character.

On acceptance of the command, a newline character is returned. Some commands preceded this with the character  '0' (success) or '1' (fail), or the requested data.

A command which is not understood will return "What is " followed by the command.

Internal PC

If the internal PC is installed and connected, the VSC900 will bleep twice on pressing the front panel power switch to ON, otherwise it will bleep once, indicating it is to be controlled by an external PC. The firmware will then start.

If neither PC is detected, there will be three bleeps, you will need to cycle the VSC80 power after reconnecting.

If the power switch is held down, there will be a series of rapid warning bleeps, at the end of this there will be a long bleep,  the VSC900 will then immediately power down.

If the internal PC is shut down from Windows, the VSC900 will power down, after a long bleep, usually 10-20 seconds after the monitor signal is lost.

Contents

General Commands	3

Power Monitoring	5

Power Supply Control (Testing Only)	6

Flood PCBs  - Left and Right	7

Multi-Spectral Flood (for VSC900)	9

Spot PCB	11

Lamp Tray PCB	12

Transmitted light PCB	15

Lighting Intensity Control	16

Motors	17

Anti Stokes LED PCB	18

Anti Stokes Flash Tube PCB [Legacy]	19

Phosphorescence	20

EEPROM Storage	21

VISCA Command and Inquiry	22

Reduced Specification Machines	23

XY Stage Control	24

I2C Address	27

Diagnostics, etc. [WIP]	28

Change History	29

V1.02.00 – 28/06/2023 - MD	29

V1.01.00 – 26/06/2023 - MD	29

V1.00.00 – 22/05/2023 - NA	29

V00.99.01 –12/05/2023	29

V00.99.00 –12/05/2023	30

V00.20.00 –13/03/2023	30

V00.19.00 – 06/03/2023	30

V00.15.00 – 17/02/2023	31

V00.12.00 – 08/02/2023	31

V00.09.00 – 20/10/2022	31

V00.06.00 – 22/08/2022	32

V00.03.00 - 19/07/2022	32

# General Commands

OFF (If#)

Turns all LEDS/Lights off.

BUILD (Vc?)

Returns the VSC900 controller firmware build number.

TYPE

Returns machine type (VCS90, VSC900, etc)

DATE (Vd?)

Returns the VSC900 controller firmware build date.

ENUM (En?)

Returns a comma separated list of connected PCBs by interrogating the I2C bus:

V900LAMPTRAY, LAMPTRAY, LFLOOD, RFLOOD, SPOT, TRANS, ASTOKES, ZOOM, EEPROM, KEYPAD, ERROR, TEMP, XYSTAGE, LMS, RMS

Note devices are only available for use after a main controller reset. (which initialises the I/O devices on the PCBs). If ERROR appears in the list, there is a possible I2C bus error.

SSTATUS (S??)  (motor fault bits set if failed to find the home sensor)

Returns the status of the motors.

0x01 Camera filter motor not yet home.

0x02 V 80 Close up lens motor not yet home. (Back Focus in V800)

0x04 One or more motors are moving.

0x08 V 80 Coax motor not yet home. (Not used in V800)

0x10 Camera filter motor fault.

0x20 V 80 Close up lens motor fault.  (Back Focus in V800)

0x80 V 80 Coax motor fault. (Not used in V800)

LSTATUS (Y??)

Returns flap and safety relay states (open=0, closed=1).

0x01 Front flap sensor.

0x02 Left flap sensor.

0x04 Right flap sensor.

0x08 Debugging - Ignore bit

0x10 Debugging - Ignore bit

0x20 Debugging - Ignore bit

0x40 Safety relay #1.

0x80 Safety relay #2.

Unsafe condition is 0, safe condition is 1 ( hence 0b11XXX111 = all OK)

TEMP

Returns the temperature reading of the temperature sensor mounted on the Spot PCB.

BUTTON  1

Front panel button LED will be OFF when powered on (default). Option is saved in EEPROM.

BUTTON  0

Front panel button LED will be GREEN when powered up. Option is saved in EEPROM.

BUTTONREAD

Returns the front panel button LED EEPROM (0 = GREEN, 1 = OFF, -1 = default-equivalent to GREEN)

Bl#

Starts the firmware boot load process. Details as for VSC6000/8000.

You may need to add a 2-3 second delay between opening the FTDI driver and sending this command.

Note: If boot load fails or is interrupted, link pins 3 and 5 on the PROG connector and  use the Windows BOOTLOAD_app.  Remove the link after the firmware update.  Note that a parameter following BL# is used to put that named device into a boot load mode.  Devices available are LAMPTRAY and XYSTAGE.  The VSC900 will respond with ‘<’ to indicate that the device is being erased and ‘>’ once it has been erased and is now in the bootloader waiting for Intel hexadecimal data.  Hexadecimal data lines start with a colon.  On completion, the devices application layer is automatically started.

WARNING: Do not press the front panel on-off button or remove power during a bootload process!

Z#~

Resets the main V80 microcontroller. You are advised to close all USB devices connected to the hub, since the power will be removed from the hub within 250 milliseconds. You can increase this time delay with Ky=n which will set the default time delay to n milliseconds.

Note there is no newline character returned after this command.

BEEP a, b

Sounds the main controller piezo buzzer. The pitch half period is a microseconds, the duration is b milliseconds. BEEP 1073, 200 will sound B flat above middle C for 200 milliseconds.

BEEPX

A variation on the above. No parameters required. (from BWV 785)

SE=1

If the FCB camera returns an error packet, a short beep will be heard. Use this command to suppress the beeps. SE=0 will enable beeps, this is the default power on setting. See the FCB manual for an explanation of VISCA error packets.

Kt=n

Sets the delay time between issuing a processor reset instruction and the reset.

This delay time is saved in EEPROM. Also applies to the Kill command.

Kt?

Reads the reset delay time.

KK​

Sends a 'kill' command to the main VSC80 microcontroller after the delay time set by Kt. This has the same effect as holding down the front panel button for 2-3 seconds.

Note there is no newline character returned after this command.

# Power Monitoring

The VSC900 has power monitoring capability on the main 12V power rail. This can be used to take indicative measurements of the system load for monitoring, testing and fault finding purposes. The values are NOT calibrated on a system-by-system basis, so the accuracy of the measurements may vary. It should also be noted that for VSC900s embedded PCs, the embedded PC power will be included in the measurements, this is likely to cause additional fluctuations to the results.

READVOLTS

Returns an integer value in millivolts (mV) indicating the bus voltage.

READAMPS

Returns an integer value in milliamps (mA) indicating the current draw.

READWATTS

Returns an integer value in milliwatts (mW) indicating the power.

# Power Supply Control (Testing Only)

Defaults to on at reset/power up.

K3=1	3V power on

K3=0	3V power off

K5=1	5V power on

K5=0	5V power off

Kc=1	Camera power on

Kc=0	Camera power off

Ut=1	FTDI interface on

Ut=0	FTDI interface off

U2=1	USB 2 hub enabled

U2=0	USB 2 hub disabled

U3=1	USB 3 hub enabled

U3=0	USB 3 hub disabled

ENABLESHUTDOWN=n (VSC80i only)

If n =1 will send a shutdown signal to the internal PC when the front panel button is pressed. This setting is not remembered and will need to be sent at power on if required. When the VSC80i firmware has detected the power down state of the internal PC it will also switch off.

The front panel button must be released immediately else it will remove the power to the internal PC within a few seconds.

# Flood PCBs  - Left and Right

There are two flood PCBs, each consisting of three UV LEDs, seven White LEDs and 8 IR LEDs (4 wavelengths total).

The White LEDS are used for flood lighting but also double up as OVD sources (indexes OVD 1-7 and 23-29). Two white LEDs are switched on per PCB when used for floodlighting. Only a single LED is lit at a time when used as OVD light sources (see Lamp Tray PCB section for more details).

UV365 n x (UV3)

Turns on the UV LEDs n at intensity x. The intensity is on a scale of 0-1023. The LED positions are shown in the table below.

| Index (n) | Led No | Mode |
| --- | --- | --- |
| 1 | 1 | Single |
| 2 | 2 | Single |
| 3 | 3 | Single |
| 4 | 1, 2, 3 | Flood Mode |
| 5 | 1, 3 | Left and Right Mode |

Typically, n=4 for flood lighting, the other options are primarily intended for test/diagnostic purposes.

IRW 110011 n

Turns on a combination of IR and white floods LEDs using a binary sequence of 6bits. The intensity value n is common for all LEDs and will accept values 0-1023.

The order of bits from left to right is as follows:

- 780nm

- 850nm

- 940nm

- 980nm

- White 1

- White 2

For example, the command to turn on 940nm, 980nm, White 1 and White 2 at intensity 400 would be:

IRW 001111 400

FLOODDIMALL n (FDA)

Dims all the flood PCB light sources (White, IR, UV and MS). Intended to be used to trim light level once a light source has already been switched on.

FLOODDIMWHITE n (FDW)

Dims only the white LEDs on flood PCB. Intended to be used to trim light level once a light source has already been switched on. Particularly when used as fill lighting with UV light sources.

IR3D n x

Used to switch on single IR LEDs in the four corners of the VSC900, primarily for generating IR source images for the 3D surface mapping tool. Turns on a single IR LED n at intensity x. The intensity is on a scale of 0-1023. The wavelength index is shown in the table below.

| LED No | Wavelength (nm) | Corner Location |
| --- | --- | --- |
| 1 | 980 | Left Front |
| 2 | 940 | Left Front |
| 3 | 850 | Left Front |
| 4 | 780 | Left Front |
| 5 | 980 | Left Rear |
| 6 | 940 | Left Rear |
| 7 | 850 | Left Rear |
| 8 | 780 | Left Rear |
| 9 | 980 | Right Rear |
| 10 | 940 | Right Rear |
| 11 | 850 | Right Rear |
| 12 | 780 | Right Rear |
| 13 | 980 | Right Front |
| 14 | 940 | Right Front |
| 15 | 850 | Right Front |
| 16 | 780 | Right Front |

*Corner location assumes that the user is looking into the machine from the front flap.

# Multi-Spectral Flood (for VSC900)

There are 2 multispectral flood PCBs. They make use of the 365nm and IR flood LEDS combined with 20 additional wavelengths. This gives a total of 25 useable wavelengths from UV 365nm up to IR 1050nm. Only 1 wavelength may be switched on at a time.

MS n x

Turns on a multi spectral LED n at intensity x. The intensity is on a scale of 0-1023. The wavelength index is shown in the table below.

| LED No | Wavelength (nm) | Filter | Driver |
| --- | --- | --- | --- |
| 1 | 365 | VIS/UV | UV |
| 2 | 385 | UV | MS |
| 3 | 405 | VIS | MS |
| 4 | 425 | VIS | MS |
| 5 | 450 | VIS | MS |
| 6 | 475 | VIS | MS |
| 7 | 500 | VIS | MS |
| 8 | 530 | 495 | MS |
| 9 | 560 | 495 | MS |
| 10 | 600 | 550 | MS |
| 11 | 620 | 570 | MS |
| 12 | 640 | 590 | MS |
| 13 | 660 | 610 | MS |
| 14 | 680 | 630 | MS |
| 15 | 700 | 665 | MS |
| 16 | 730 | 695 | MS |
| 17 | 750 | 695 | MS |
| 18 | 780 | 725 | IR |
| 19 | 810 | 780 | MS |
| 20 | 850 | 780 | IR |
| 21 | 890 | 850 | MS |
| 22 | 910 | 850 | MS |
| 23 | 940 | 850 | IR |
| 24 | 980 | 850 | IR |
| 25 | 1050 | 850 | MS |

365nm (Index 1) is identical to the original UV365 command, however only 2 of the 3 UV LEDs are turned on.

The FLOODDIMALL (FDA) may be used to dim the MS LEDs once they have been switched on.

If this board is fitted in the ENUM command will return “LMS” and “RMS” for left and right respectively.

MSWHITE n x

Turns on a white LED n at intensity x. The intensity is on a scale of 0-1023. This command is primarily intended for testing/diagnostic purposes. The LED positions are shown in the table below.

| Index (n) | Led No | Mode |
| --- | --- | --- |
| 1 | 1 | Single |
| 2 | 2 | Single |
| 3 | 3 | Single |
| 4 | 4 | Single |
| 5 | 5 | Single |
| 6 | 6 | Single |
| 7 | 7 | Single |
| 8 | 1, 7 | Flood Mode |
| 9 | 1, 2, 3, 4, 5, 6, 7 | All On Mode |

MSSIDE n

Selects left, right or both sides of the multispectral flood lighting for testing purposes. The table below shows the valid n values.

| Index (n) | Side/s |
| --- | --- |
| 0 | Both (default) |
| 1 | Left |
| 2 | Right |

By default, both multispectral flood PCBs will be used when any flood or multispectral flood commands are used. However, for testing purposes it may be desirable to drive one flood array at a time.

To do this an MSSIDE 1 or 2 command should be sent to select the side, followed by the lighting commands for that side. MSSIDE 0 should then be used to return to the default state. Additionally using the OFF (If#) command will also return the flood arrays to the default state of both being enabled.

# Spot PCB

Nine coloured LEDs and one White LED.

VIOLET n

Turns on violet spot LED only at intensity n. (0-1023)

BLUE, BLUEGREEN, CYAN, GREEN, AMBER, RED, DEEPRED, FARRED, WHITESPOT.

As above to turn on one only of these LEDs.

SPOT 1100000000 n

Turns on Violet and Blue spots  LEDS together at intensity n. There must be exactly ten 1s and 0s in the first argument string to turn on any number of LEDs corresponding to the LED order from left to right of: Violet, Blue, Blue-Green, Cyan, Green, Amber, Red, Deep Red, Far Red and White.

If n=0, LEDs will turn off.

If more than 2 LEDs are on, the intensity level will be automatically de-rated, depending on the number of active  LEDs. The maximum de-rating factor for all 10 LEDs is 0.5 . The actual intensity applied will be returned.  If P4 is subsequently used to adjust the intensity, the applied intensity, de-rated if necessary, will also be returned.

The de-rating factors are, for 1 to 10 LEDS:

1.0, 1.0, 0.90, 0.80, 0.75, 0.70, 0.65, 0.60, 0.55, 0.50

LED wavelengths are:

Violet		410nm

Blue		445nm

Blue/Green	475nm

Cyan		500nm

Green		520 nm

Amber		590nm

Red		640nm

Deep Red	660nm

Far Red	740nm

White		420nm-700nm

SPOTSHIFT n

Changes all the I2C addresses for the Spot PCB to their values before any XOR shifting is applied on the lamp tray PCB. This allows the Spot PCB to be directly plugged into the control PCB for testing purposes. The table below shows the valid n values.

| Index (n) | Shift Removed |
| --- | --- |
| 0 | No (default) |
| 1 | Yes |

By default, the spot PCB addresses will always be shifted to be compatible with the Lamp Tray PCB. This default state will be restored after a reset to power-cycle.

# Lamp Tray PCB

LVER

Returns VSC900_LAMPTRAY if lamp tray connected and running or BOOTLOAD if no application layer present on the lamp tray PCB.

LBUILD (Vl?)

Returns the lamp tray firmware build number in the form major, minor and build number.

LDATE

Returns the lamp tray firmware build date showing “Released” or “Debugging” followed by the timestamp in the form dd.mm.yyyy at HH:MM”.

Lamp Tray Light Sources:

- Sidelights - Left and Right White and IR LEDs

- Coax LED

- 50 OVD LEDs on the VSC900 (36 on the lamp tray PCB, remainder on MS PCBs)

  - Including 3 polarised light sources

- 21 OVD LEDs on the VSC80 (13 on the lamp tray PCB, remainder flood PCBs)

- Two UV LED Wavelengths (308nm and 255nm)

Lq=n

Turns on one of the LEDs in the OVD array. For example, Lq=21 will turn on LED index 21.

For V900:

- Lq=1-52 is valid (2 LEDs are duplicated)

- Lq=1–7 are LEDs on the Left MS PCB

- Lq=23-29 are LEDs on the Right MS PCB

- All remaining LEDs are on the Lamp tray PCB

For V90:

- Lq=1-22 is valid (1 LED is duplicated)

- Lq=1–4 are LEDs on the Left Flood PCB

- Lq=14-17 are LEDs on the Right Flood PCB

- All remaining LEDs are on the Lamp tray PCB

More details can be seen in the below LED assignment layouts.

When the Lq=n command is used it will turn on the selected LED using the stored EEPROM intensity. By default, this is 1023 (max intensity level) on a new machine. All OVDs have the same intensity scale of 0 (off) to 1023 (maximum). The Lq=n command will return the intensity the LED is set at.

The intensity of an OVD LED can be specified with an additional parameter. This is outlined below.

Lq=n1 n2

Turns on one of the LEDs in OVD array with a chosen intensity. For example, Lq=21 400 will turn on LED index 21 at intensity 400.

The last intensity level used for each OVD led will be saved in EEPROM with this command format. If a subsequent OVD command has no level parameter specified (using Lq=n format), then the value stored in EEPROM for each LED will be used.

Lq? n

Diagnostic command to read the EEPROM intensity level stored for a specified OVD LED index. For example, Lq? 12 might return the value 500 if that was the last specified intensity.

Polarised Light Sources (VSC900):

There are 3 polarising filters fitted to LEDs in the VSC900 OVD array. The filters are configured to reuse the following OVD LEDs as polarised light sources:

- 15/45 – Linear polarised light

- 51 – Left circular polarised light

- 52 – Right circular polarised light

The Lq commands described above can be used to control these light sources.

VSC900 OVD Configuration

VSC90 OVD Configuration

22

4	5	6	7	8	9 (21)	10	11	12	13	14

3					20					15

2					19					16

1					18					17

OVDCONF

Returns an 8 bit value which indicates which OVD related PCBs are present in the system. This can be used for debugging and to determine the supported Lq range.

Valid combinations are as follows:

- 0b00010011 (Dec: 19) - V900 Lamp Tray and V900 MS Floods (Lq 1-52)

- 0b00011100 (Dec: 28) - V900 Lamp Tray and V80 Floods (Lq 1-52, some LEDs skipped)

- 0b00101100 (Dec: 44) - V80 Lamp Tray and V80 Floods (Lq 1-22)

COAX n (Ax=n)

Turns on the Coax LED at intensity n

SIDELEFT n (Ls=n)

Turns on the left side White and IR LEDs together at intensity n.

SIDERIGHT n (Rs=n)

Turns on Right side White and IR LEDs together at intensity n.

SIDE 1100  n

Turns on one or more side lights at intensity n.

The first parameter is a string of exactly four 1s or 0s which controls the side lights in the order:

Left White - Right White - Left IR - Right IR

The example above will turn on both the left and right white LEDs and turn off both IR LEDs. This may not be a lighting requirement, but it allows individual LEDs to be tested.

UV255 n (UV1=n)

Short wavelength UV LEDs. The wavelength will be 255nm for the VSC90/900 UV LEDs.

UV LEDs are either on (n != 0) or off (n = 0)

UV308 n (UV2=n)

Medium wavelength UV LEDs. The wavelength will be 308nm for the VSC90/900 UV LEDs.

UV LEDs are either on (n != 0) or off (n = 0)

See also the UV3 command on the Flood boards for 365nm UV flood LEDs.

# Transmitted light PCB

TRANSSPOT=n X (TS=n X)

Turns on a combination of IR and white transmitted spot LEDs using a binary sequence of 4bits (n) at the selected intensity level X. The intensity value X is common for all LEDs and will accept values 0-1023.

The order of bits from left to right is as follows:

- 780nm

- 940nm

- 850nm

- White

For example, the command to turn on 780nm, 940nm and White at intensity 500 would be:

TRANSSPOT 1101 500

TRANSFLOOD=n X (TF=n X)

Turns on a combination of IR, UV and white transmitted flood LEDs using a binary sequence of 4bits (n) at the selected intensity level X. The intensity value X is common for all LEDs and will accept values 0-1023.

The order of bits from left to right is as follows:

- UV 365nm

- 780nm

- White

- 850nm

Note: Not all combinations valid. If the UV365nm bit is set all remaining bits will be ignored.

For example, the command to turn on 850nm, White at intensity 500 would be:

TRANSFLOOD 0011 500

TRANSOFF

Turns off all the translight LEDs, both Trans Spot and Flood.

TRANSCHECK (TC)

Checks the status of the translight. Responds with an integer value indicating state as below:

- 0 – Translight OK – The translight tray is inserted and ready to be used.

- 1 – Translight SWAPPED – The translight tray has been removed and re-inserted, it needs to be initialised before use. The TRANSINIT command below, should be used before issuing any new translight commands.

- 2 – Translight MISSING – The translight is currently removed from the system

TRANSINIT (TI)

Re-initialises the translight ready for use. Returns 0 if the translight tray can be found for initialisation. This command should be followed with a TRANSCHECK to make sure the translight is ready for use.

# Lighting Intensity Control

Various PCBs have their own local intensity control for the LEDs. If the intensity parameter is not given in the above commands, then the previous value will be used. Most are DACs, but the tray LED level is set by PWM. (there may be differences in level numbers if OVDs are dimmed since some are DAC and some are PWM).

Should it be required to dim individual LEDS, these are available:

P4=n

Sets the Spot board intensity.

P5=n

Sets the Lamp Tray board board intensity. (L & R side lights, Coax, OVD on Lamp tray).

# Motors

C1

Camera filter wheel.  ( There is no C2 camera filter motor fitted)

C1=n, C1?, C1>, C1<, C1h, C1z, C1o

Operates as V40 filter wheel.

C1=n moves the filter wheel to position n (0-12), by the shortest route.

C1? Returns the filter position.

To calibrate: Apply C1z to clear the calibration. Apply C1h to home the wheel.

Use C1> or C1< as required, to nudge the wheel either direction to centralise the clear filter element in the field of view. This adjustment value is stored in EEPROM.

Finally apply C1h to home the wheel, and verify that the alignment is OK.

C1h can be applied thereafter anytime. Motors must be homed from a home command at power on.

C1o is a diagnostic to read the offset value and will return a signed number as a string.

C1F

Returns filter population string. 1 = filter element is fitted, 0 = filter element blank. (future use)

C3

Close up lens.

C3=n, C3>, C3<, C3h, C3z

To calibrate:

Apply C3h	moves motor to find sensor (out)

Apply C3z	stores 80 degrees in memory. (about 350 steps)

C3=1 		Moves to above value.

C3>, C3<	Nudges above value and motor, then saves value in memory.

Thereafter use C3h  (or C3=0) to move out and C3=1 to move into the optical path.

The close up lens will not move until it is calibrated.

C4

Coax mirror Motor.

Required to be in position when Coax source is on, or parked otherwise.

There are two calibration points, one to set the filter horizontal (parked) and the second to set the filter to 45 degrees, or as required for a coax image.

Calibrate as for camera filter. Apply: C4z, C4h,  then C4> or C4< as required to set horizontal. Apply C4=1, then C4p or C4m as required for a coax image ( a suitable coax sample is required). Finally apply C4h (or C4=0)  and C4=1 to verify both positions are correct .

The coax lens will not move until it is calibrated.

C4z		Clears both the calibration points.

C4h		Home (parks) motor(horizontal).

C4>, C4<	Fine adjustment to home position.

C4=1		Move to working position (45 degrees or as required)

C4p, C4m	Fine adjustment to working  position.

C4=0		Equivalent to home (park).

Cnh will home all three motors together.

Spare motor (not connected)

C5=n, C5?, C5>, C5<, C5h, C5z

# Anti Stokes LED PCB

Antistokes LED light source is fitted in the VSC900. The LED array is split into 2 banks which can be separately controlled. The light source also has 2 power levels, DIM and HIGH. In HIGH mode the operation time of the antistokes should be limited (additional firmware and hardware guards are also implemented).

AS?

Return whether an antistokes is fitted.

- 0 – Antistokes not fitted

- 1 – Antistokes not fitted

ASVER

Returns the version of antistokes fitted in the machine.

- 0 – Not Fitted

- 1 – Flash Tube Antistokes

- 2 – LED Antistokes

AS0

Switch off the LED antistokes array

AS1

Switch on bank 1 of the antistokes LED array in low power mode. AS0 command should be used first to turn off the array if it was previously used in high power mode.

AS2

Switch on bank 2 of the antistokes LED array in low power mode. AS0 command should be used first to turn off the array if it was previously used in high power mode.

ASDIM ASD

Switch on the antistokes array in dim mode. AS1 and AS2 must have been sent prior to this command.

ASHIGH ASH

Switch on the antistokes array in high power mode. AS1 and AS2 must have been sent prior to this command.

# Anti Stokes Flash Tube PCB [Legacy]

Photo flash tube and charging circuit.

CHARGE=n (Fl+)

If n is 1, turns on the charging circuit (allow approximately 5 seconds to fully charge).

CHARGE=0 or Fl-  Turn off the charging circuit.  Firing the flash will also turn off the charging circuit. This function can also be used to remove the freeze frame state, the camera then outputs live video. Note the charging circuit should also turn off when a fully charged state has be detected by the charging electronics.

FSTATUS (Fl?)

Returns 0 when fully charged, else returns 1.

FLASH (Fl@)

Fires the flash, then freezes the frame after the number of frame counts set by Fd=n . Use Fl- to turn freeze frame off. Fd is set to default to 1, which probably works. If not try different values. The flash function should return zero. If non-zero is returned it indicates that the VSC80 could not detect the camera's horizontal sync pulse and a timeout condition was detected.

# Phosphorescence

LL@ (Phosphorescence?)

Waits for the next camera frame sync pulse then delays by the number of milliseconds specified by Gd=n, then switches off all LEDS with a Global Enable signal. This function will capture and freeze the video after a further frame count set by Ld=n. Where n can be 0, 1, 2... selected by experiment. This function should return zero. If non-zero is returned it indicates that the VSC80 could not detect the camera's horizontal sync pulse and a timeout condition was detected.

After use, you may need to re-enabled Global enable with Ge=1 to turn LEDs back on, unless there is a LED change or LED on command, which should automatically re-enable the Global enable. You may also need to clear the controller buffer which holds the VISCA reply packet using the command FCB, which returns and clears the return buffer in the microcontroller. For normal VISCA commands, the buffer content is returned and cleared automatically.

Gd? and Ld? Will return the above settings. One frame is 40 milliseconds. Gn should not normally exceed 40. The camera frame rate may need to be 1/25 (40 milliseconds) for Anti-Stokes and phosphorescence to operate correctly.

# EEPROM Storage

The VSC900 has a user available 32k EEPROM organised as pages of 64 bytes.

Xb=n

Selects the page number (0-510)

Xr?

Read 64 bytes from the current page. After sending this command, read back 64 more bytes. A newline character is not returned after the 64 bytes are received, so make sure exactly 64 bytes are requested.

Xw=string

Writes a string up to 64 characters in length to the current page. If the length is less than 64, the page will be zero padded. String may contain CR characters, (0x0d) but not LF (0x0a) which terminates the command. A read verify is done after the EEPROM write, this command will then return the number of bad bytes, which hopefully will be zero.

Xw+

Writes 64 bytes to the current page:

1. Send Xw+

2. Wait for a newline character to be returned.

3. Write exactly 64 bytes.

4. Read back two bytes. The first byte should be null (0x00) followed by a newline. If the first byte is not null, an error occurred. The write data is checked against a read of the data, the first byte returned is the number of errors detected.

Note that write data should be limited to 7 bit ASCII.

Machine settings (motor calibration, firmware options etc.) are stored in a different area of EEPROM inside the microcontroller. Firmware upgrades do not overwrite any EEPROM data.

# VISCA Command and Inquiry

You can send and receive VISCA packets to the camera by sending the VISCA command packet directly to the V80 controller. The command and inquiry packets are as given in the camera Technical Manual, followed by a newline character.

For example “8101040102FF\n” will turn on the ICR (near infra red) mode. The returned packet should be 9041FF9051FF (acknowledge packet followed by completion packet)

Inquiry packets are handled in the same manner, “8101044AFF\n” will return the shutter value. For example “905000000005FF” is returned when the shutter is at index position 5 (1/25).

To prevent lock up in case a return packet is not received from the camera, zero will be returned after a timeout period. This timeout can be set by TO=n, the units of n are 65 milliseconds. The default setting is 8 (8*65 ~= 0.5 seconds).

FCB

Will return the last reply from the camera, which is retained by the V80 controller. If this buffer has already been read (it usually has in most cases), zero will be returned.

Reading this value at 1 second intervals at start up may indicate when the TWIGA board has completed sending VISCA initialisation commands. Wait for 2 consecutive zero responses.

# Reduced Specification Machines

For future reduced spec machines, the following commands apply.

Op?

Returns the machine type.

0: Full specification VSC900 (FS).

C1p

Returns a 19 character string. '1' indicates the camera filter in that position is present, '0' indicates there is a blanking plate in these positions.

FS machines will return  “1111111111111111111”.

Shp

Returns a 10 character string. '1' indicates that the spot wavelength LED is fitted and  '0' the LED is not fitted.

FS machines will return  “1111111111”.

As?

Returns 1 if the Anti-Stokes unit is fitted.

The above information is stored in EEPROM memory.

The following may be used to configure a machine.

Production will a Windows utility program to program the settings below:

SETMACHINE n

For FS machines n should be set to 255. For M1 machines, set n to 1, for M2 machines set n to 2.

SETFILTER  1111111111111111111

Will set the filter wheel to type FS.

SETSPOT 1111111111

Will set the spot LEDS to type FS

SETSTOKES n

For machines with Anti-Stokes fitted,  n should be 255, for machines without Anti-Stokes, n should be set to 0.

# XY Stage Control

XYVER? (VX?)

Returns the XY stage firmware version as a string in Major.Minor.Build format.

(For example: Request: VX?, Response: 1.02.0)

XYDATE?

Returns the XY stage firmware build date showing “Released” or “Debugging” followed by the timestamp in the form dd.mm.yyyy at HH:MM”.

XYRESET (XR)

Resets the XY stage microcontroller. Not required in typical operation.

XYSTATUS? (XST?)

Gets the XY stage status as a 16bit integer value (2 bytes). Each bit will represent a different flag.

- To get the status byte, use XYSTATUS? or XST?. (For example: Request: XST?, Response: 2405)

- The value will then need to be interpreted as binary flags, for example “2405” will create the following set of binary flags “0000 1001 0110 0101”.

Status flag bit definitions:

- Bit 12-15: Not used

- Bit 11: Bit is set if XY stage is uncalibrated

- Bit 10: X or Y limit was sensed

- Bit 9: X limit was sensed

- Bit 8: Y limit was sensed

- *Bits 8-10 are cleared by clearing the “sensor flag state” byte

- Bit 7: Error in X home sensor during homing

- Bit 6: Error in X limit sensor during homing

- Bit 5: Error in Y home sensor during homing

- Bit 4: Error in Y limit sensor during homing

- Bit 3: X motor homing in progress

- Bit 2: Y motor homing in progress

- Bit 1: X motor is moving (bit is clear when homing)

- Bit 0: Y motor is moving (bit is clear when homing)

XYPOSX=n (X=n), XYPOSY=n (Y=n)

Sets the absolute position of the XY stage in the X or Y axis.

- To set an absolute X position, use XYPOSX=n or X=n. (For example: XYPOSX=1200)

- To set an absolute Y position, use XYPOSY=n or Y=n. (For example: XYPOSY=1200)

To find the maximum and minimum values for this axis, use the working position commands (see below).

XYPOSX? (X?), XYPOSY? (Y?)

Gets the current absolute position of the XY stage in the X or Y axis.

- To get the current X position, use XYPOSX? or X?. (For example: Request: X?, Response: 230)

- To get the current Y position, use XYPOSY? or Y?. (For example: Request: Y?, Response: 230)

XYFWD=n (XFW=n), XYBACK=n (XBK=n), XYLEFT=n (XLF=n), XYRIGHT=n (XRH=n)

Moves the XY stage a defined number of steps forward, back, left, and right.

- To move forward, used XYFWD=n or XFW=n. (For example XFW=200)

This movement is made relative to the current position and will update the coordinate location accordingly. Movement will be stopped at the limit switches.

XYSPEED=n (XSP=n)

Sets the speed setting for the XY stage.

- To set the speed use XYSPEED=n or XSP=n. (For example: XYSPEED =100)

XYSPEED? (XSP?)

Gets the current speed setting of the XY stage.

- To get the current speed setting, use XYSPEED? or XSP? (For example: Request: XSP? Response: 120)

XYACCEL=n (XA=n)

Sets the acceleration setting for the XY stage.

- To set the acceleration use XYACCEL =n or XA=n. (For example: XYACCEL=100)

XYACCEL? (XA?)

Gets the current acceleration setting of the XY stage.

- To get the current acceleration setting, use XYACCEL? or XA? (For example: Request: XA?) Response: 120)

XYSTSPEED=n (XSS=n)

Sets the start speed setting for the XY stage.

- To set the start speed use XYSTSPEED=n or XSS=n. (For example: XYSTSPEED =100)

XYSTSPEED? (XSS?)

Gets the current start speed setting of the XY stage.

- To get the current start speed setting, use XYSTSPEED? or XSS? (For example: Request: XSS? Response: 120)

XYTRAVELX? (XR?), XYTRAVELY? (YR?)

Gets the X or Y max travel in mm. Not yet implemented

XYWPX0? (X0?), XYWPY0? (Y0?)

Gets the working position 0 of the X or Y motor in steps. This is the lower end of the working range.

- To get working position 0 of the X axis, use XYWPX0? (For example: Request: XYWPX0? Response: 504)

- To get working position 0 of the Y axis, use XYWPY0? (For example: Request: XYWPY0? Response: 489)

XYWPX1? (X1?), XYWPY1? (Y1?)

Gets the working position 1 of the X or Y motor in steps. This is the upper end of the working range.

- To get working position 1 of the X axis, use XYWPX1? (For example: Request: XYWPX1? Response: 5208)

- To get working position 1 of the Y axis, use XYWPY1? (For example: Request: XYWPY1? Response: 4230)

XYSENSORS? (XS?)

Gets the state of all sensors combined as a byte. Only the lower 4 bits are used to indicate the state of the sensors.

- To get the sensor byte, use XYSENSORS? or XS?. (For example: Request: XS?, Response: 10)

- The value will then need to be interpreted as binary flags, for example “10” will create the following set of binary flags “1010”.

Sensor flag bit definitions:

- Bits 4-7: Not used

- Bit 3: X home sensor status (bit is set when X is home)

- Bit 2: X limit sensor status (bit is set when X is at limit)

- Bit 1: Y home sensor status (bit is set when Y is home)

- Bit 0: Y limit sensor status (bit is set when Y is at limit)

XYFLAGS=n (XF=n)

Clear the limit sensor flag states in the status word (bits 8-10) and XYFLAGS by writing 0 to this byte. (For example, XYFLAGS=0).

XYFLAGS? (XF?)

Gets the status of the limit switch flags as a byte. These are set if a limit switch has been hit. Only the lower 2 bits are used.

- To get the limit flag byte, use XYFLAGS? or XF?. (For example: Request: XF?, Response: 3)

- The value will then need to be interpreted as binary flags, for example “10” will create the following set of binary flags “0011”.

Limit switch flag bit definitions:

- Bits 2-7: Not used

- Bit 1: X limit sensor activated

- Bit 0: Y limit sensor activated

XYHOME (XH)

Forces a motor home move for X and Y motors. This action will take several seconds to complete. No other commands should be sent to the XY stage until this is complete. Check the status byte to see whether this has been completed.

XYLIMITOP=n (XL=n)

Set the action of limit switch operation.

Limit switch operation bit definitions:

- Bits 1-7: Not used

- Bit 0: If this bit is set, home the XY stage when the X or Y limit sensors are activated

XYLIMITOP? (XL?)

Get XY stage operation when the X or Y limit sensor is activated.

XYTEST (XT)

Run a sequence of test movements. This will take approximately a minute to complete, depending on speed setting. The movements are as follows:

- A set speed by running the XY motors separately in the left to right and front and back directions across the XY stage.

- A set speed by running the XY motors together in the left to right and front and back directions across the XY stage.

- A set acceleration by running the XY motors separately in the left to right and front and back directions across the XY stage.

- A set acceleration by running the XY motors together in the left to right and front and back directions across the XY stage.

- A set acceleration by moving the XY stage to the home position and back

- A set acceleration by finally moving the XY stage to the home position to end the test routine

XYCAL (XC)

Calibrates the XY stage home position.

# I2C Address

Various components are connected to the main processor with an I2C bus.

You can check the acknowledge from these devices by sending the command I2C n

where n is the 8bit WR device address.

Values for n are (use the hex or decimal form):

All these should return zero when they respond, except Error which is an address which does not exist. If this returns zero, the data line may be stuck.

A missing or faulty device usually responds with -2 (try a re-boot of the system!)

# Diagnostics, etc. [WIP]

DAC0 n

DAC1 n

Sets Flood DACs to n

I2C byte sequences can be sent directly from the command box (if you can work them out!). Mainly for firmware/hardware bash testing:

OUT 72 20 -73 r

This sends the I2C byte sequence:

72 (0x48)	The flood I/O expander Write address.

20 (0x14)	The address of the Latch A register.

73 (0x49)	The I/O flood expander Read address, preceded by a restart

state  ( - ).

r		 Byte read request. Returned byte is in the Reply box

(reads the contents of register 0x14 on the flood PCB expander)

OUT 72 20 n

Writes n to the I/O expander Latch A register.

I2C address

Writes address byte to I2C device. Returns zero if  device acknowledge, else returns non-zero. Address may be decimal or hex if preceded with 0x

# Change History

## V1.02.01 – 04/07/2023 - MD

New Commands:

- LDATE returns date stamp of the lamp tray firmware (requires lamp tray firmware update).

- XYDATE? returns date stamp of the XY stage firmware (requires XY stage firmware update).

Changed Commands:

Removed Commands:

Other Notes:

## V1.02.00 – 28/06/2023 - MD

New Commands:

Changed Commands:

Removed Commands:

Other Notes:

- A further correction required in filter wheel stepper motor driver – no evidence in operation, but fault noticed in follow-on review of code that speed parameters were being used prior to their calculation.  It appears that by good fortune the previous iteration of the speed value happens to be preserved and used instead!

## V1.01.00 – 26/06/2023 - MD

New Commands:

- OFF Duplicate of IF# command to switch everything off - already mentioned in earlier versions, but not actually available until this version.

Changed Commands:

Removed Commands:

Other Notes:

- Correction to increase the drive current for filter wheel and camera lens stepper motors - they were being commanded with the high driver pin active (low), but then this pin was being released during the actual movement of the stepper motor!

## V1.00.00 – 22/05/2023 - NA

New Commands:

Changed Commands:

Removed Commands:

Other Notes:

- Release version produced.

## V00.99.01 –12/05/2023

New Commands:

Changed Commands:

- BUTTON Flipped the logic for consistency with existing product.

Removed Commands:

Other Notes:

## V00.99.00 –12/05/2023

New Commands:

- BUTTONREAD Provides EEPROM selection for button LED.

- IR3D n x Used to switch on single IR LEDs in the four corners of the VSC900 for 3D imaging.

Changed Commands:

- BUTTON Changed so restart is NOT required to change selected LED colour. May affect existing software because a bug in the firmware prevented the LED colour changing until the unit was restarted.

Removed Commands:

- Most Lighting Intensity Control commands removed. These were not used in software and did not apply to the MS floods PCBs. Spot and Lamp tray (P4+5) commands left in case later required.+

- VSC800 Lens Commands – Removed as these were not used in software and did not apply to the VSC900.

- Lamp Tray I2C Commands section

Other Notes:

- I2C Table – Updated for VSC900

- Antistokes LED – Command set added to this document.

- Reduced Spec – Updated for VSC900

- Phosphorescence – Split into separate section

## V00.20.00 –13/03/2023

New Commands:

- MSWHITE Turns on a white LEDs for testing purposes.

- MSSIDE Selects left, right or both sides of the multispectral flood lighting for testing purposes.

- SPOTSHIFT Changes all the I2C addresses for the Spot PCB to their values before any XOR shifting is applied on the lamp tray PCB.

Changed Commands:

- UV365 n (UV3) Changed to take 2 arguments, index, and intensity.

  - For flood mode UV365 4 x should be used

Removed Commands:

- I2C 70 - Will return 0 if the trans base is connected or non-zero if disconnected.

Other Notes:

## V00.19.00 – 06/03/2023

New Commands:

- READVOLTS Reads the bus voltage.

- READAMPS Reads the current.

- READWATTS Reads the power.

Changed Commands:

Removed Commands:

Other Notes:

## V00.15.00 – 17/02/2023

New Commands:

- TRANSCHECK (TC) Checks the status of the translight.

- TRANSINIT (TI) Re-initialises the translight.

Changed Commands:

Removed Commands:

- DV?

- DV

Other Notes:

## V00.12.00 – 08/02/2023

New Commands:

- FLOODDIMALL n (FDA) Dims all flood LEDs

- FLOODDIMWHITE n (FDW) Dims only the white flood LEDs

Changed Commands:

- MS updated to have control of 25 wavelengths. Command now takes index and intensity arguments.

Removed Commands:

Other Notes:

## V00.09.00 – 20/10/2022

New Commands:

- Added list of XY commands

- TRANSFLOOD=n X (TF=n X)

Changed Commands:

- ENUM (En?) will return V900TRANS if VSC900 type translight PCB is present. It will return V80TRANS for a V80 styles translight.

- TRANSSPOT command has new arguments and short command has changed to (TS)

Removed Commands:

- T1=n, T2=n, T3=n, T4=n

- TRANSWHITE n (Ix=n)

- TRANSUV n (Ux=n)

Other Notes:

- FTDI Serial for all devices will now be “VSC900” instead of “VSC80”

## V00.06.00 – 22/08/2022

New Commands:

Changed Commands:

- UV2 has changed to UV365 (UV3)

- UV254 n (Uf=n) has changed to UV255 n (UV1=n)

- UV312 n (UV=n) has changed to UV308 n (UV2=n)

Removed Commands:

- PWMP n

- IRX 1100 n

- IR n

- W12 n

- IR780, IR850, IR940, IR980, W1 and W2

Other Notes:

## V00.03.00 - 19/07/2022

New Commands:

- OVDCONF command added to report the OVD hardware config

Changed Commands:

- SLAVE (Vv?) has changed to LBUILD (Vl?)

- BUILD (Vs?) has changed to BUILD (Vc?)

- ENUM (En?) will return V900LAMPTRAY if VSC900 style lamp tray PCB is present

- Lq now also supports a range of 1-52 for the VSC900 OVD array (as well as 1-22 for V80/V90 OVD arrays)

Other Notes:

- VSC900 Lamp Tray firmware V00.01.00+ is required for the OVD and flap bits to work correctly.